FRX® Wood Specification

The following paragraphs are for insertion into a section of generic specifications or generic/proprietary specifications.

Part 1 – General

1.01 Product Identification

A. All lumber and plywood designated to be exterior fire retardant treated shall be pressure impregnated with FRX chemicals and shall have a flame spread rating of 25 or less when tested in accordance with ASTM E-84, “Standard Test Method for Surface Burning Characteristics of Building Materials.” Also, when test conditions are extended to 30 minutes, the flame spread should not progress more than 10 1/2 feet beyond the centerline of the burners, with no evidence 
of significant progressive combustion.

B. Each piece of fire retardant treated lumber and plywood shall be manufactured under independent third party follow-up inspection service, and each piece shall bear the appropriate qualified inspection agency’s label indicating surface burning characteristics in the 30 minute ASTM E-84 flame spread test. Each piece will be labeled indicating kiln dried after treatment (KDAT) and identifying the treating company and location.

C. In addition, there shall be no increase in 
the listed classification when tested after ASTM D-2898 “Standard Method of Accelerated Weathering of Fire Retardant Treated Wood for Fire Testing.”

Part 2 – Products

2.01 Lumber

Provide lumber of the appropriate grade and species as specified by the design criteria for the intended application.

2.02 Plywood

Provide plywood of the appropriate grade and species as specified by the design criteria for the intended application.

2.03 Siding

Provide siding of the appropriate grade and species as specified by the design criteria.

2.04 Fire Retardant Treatment

A.  Treatment shall be FRX chemicals manufactured by Chemco Acquisition, Ferndale, WA.

B.   All exterior fire retardant treated wood shall be kiln dried after treatment to a moisture content of 19% for lumber and 15% for plywood.

C.   Kiln drying after treatment shall be monitored by an approved third party agency, and the identification label on each piece of wood will so indicate.

D. The fire retardant formulation must be free of halogens, sulfates, chlorides and ammonium phosphate.

Part 3 – Execution

3.01 Field Cuts

Do not rip or mill FRX fire retardant treated lumber. End cuts, drilling holes and joining cuts are permitted. Plywood may be cut in any direction.

3.02 Application

A.   FRX fire retardant treated lumber and plywood used in structural applications shall be applied according to strength tables provided by Arch Wood Protection.

B.   FRX is a non-leachable fire retardant treatment and may be installed with direct exposure to precipitation, however, it cannot be substituted for preservative treated wood.

C.   FRX wood shall not be used in enclosed roofing applications.

D.   FRX wood shall not be used in direct contact with the ground.

E. Exterior fire retardant treated lumber and plywood shall use design value adjustments and span ratings as published by Arch Wood Protection.